| NORMAL PROCEDURES – CESSNA 180 K |  |  |  |
| --- | --- | --- | --- |
| Before Start Passenger Briefing – PIC, Seatbelt ON Talking, Exits, Traffic Fuel Selector – BOTH Radios & Electrical – OFF Brakes – TEST Circuit Breakers Starting Engine Cowl Flaps – OPEN Master – ON Fuel - BOTH Lights – Beacon Flaps – Up Mixture – RICH Propeller – HIGH RPM Throttle – CLOSED Carb Heat – OFF Prime & LOCK Prop – CLEAR Ignition – START Throttle – CRACK OPEN Oil Pressure Throttle – 1000-1200 RPM Mixture – LEAN After Start Ammeter Avionics – ON Transponder – 1200/STBY Circuit Breakers Radios – SET, ATIS Altimeter – SET D.G. – Set A.I. – CHECK ERECT Lights – NAV/ Strobe/ Lndg Taxi Clearance Brakes – TEST Shutdown Avionics – OFF Mags - Check Mixture, Mags, Master – OFF Control Lock – INSTALL Pitot Cover - INSTALL Fuel Selector – As Req’d Chocks – As Req’d | Run Up Brakes – SET Doors – CLOSED/ LOCKED Trim – SET Controls – FREE, CORRECT Instruments – CHECK Gas – BOTH Alitmer/Ammeter Mixture – RICH Runup Throttle- 1700 RPM; Note M.P. Engine Instruments – CHECK Prop – MP, RPM, OIL Mags – 150/50 Max Carb Heat – TEST Throttle – Idle to 1000 RPM Mixture – LEAN D.G. – SET, A.I. ERECT Radios – SET Pre-Takeoff Doors/Windows – LOCKED Cowl Flaps – OPEN Carb Heat – OFF Throttle – 1000 RPM Flaps – 20 Time – SET / NOTE Lights – ON, Trans – ALT Mixture – BEST POWER TO BRIEF Normal Takeoff Throttle – FULL Prop – PER TEMP Airspeed – ALIVE Climb – 80 KTS Obstacle – 20° & 57 KTS Climb Airspeed – 90-95 KTS Power - 23 IN and 2300 RPM Mixture – LEAN ABOVE 5000’ Engine – IN THE GREEN Cowl Flaps – OPEN Cruise Engine – 22.5 IN, 2350 RPM MIX – LEAN/RICH 50° OF PEAK Cowl Flaps – CHT – 350-390 |  | Descent Cowl Flaps - CLOSED Power – Reduce 2 IN/MIN MAX Mixture – ENRICHEN Carb Heat – AS REQ’D Engine – IN THE GREEN …Before Landing FLAPS – 20° - 40 ° Base – 65 KTS, Flaps 20° Final – 55 KTS, Flaps 40° Carb Heat – ½ OPEN Gas – BOTH Undercarriage – Check Brakes Mixture – RICH Prop/Power – HIGH RPM Seatbels / Switches Silence FEET Balked Landing Power – MAX Carb Heat – OFF Flaps - 20° until 55 KTS Cowl Flaps - OPEN After Landing Carb Heat – OFF RPM – 1000, Mix – LEAN Transponder - 1200 / STBY Lights – AS REQ’D Trim – SET FOR TAKEOFF Flaps – RETRACT AIRSPEEDS VSO – Flaps Down - 49 KTS VS1 – Flaps Up - 55 KTS Vx – Best Angle - 80 KTS Vy – Best Rate - 57 @ 20° Vfe – Flaps 10° - 120 KTS Vfe – Flaps 20°-40° - 90 KTS Va – 2800 lbs - 109 KTS - 2315 lbs – 100 KTS - 1900 lbs – 90 KTS VNO – Max Cruise - 139 KTS VNE – Never Exceed – 169 KTS BEST GLIDE (FLAPS UP) 2800 lbs – 75 KTS 2400 lbs – 70 KTS 2300 lbs – 68.5 KTS |
| PRE-FLIGHT PROCEDURES – CESSNA 180K |  |  |  |
| COCKPIT Airworthiness, Regist., Radio Lic., POH, W&B Set Clock Check VOR log for currency ELECTRICAL SWITCHES – Verify OFF AVIONICS MASTER SWITCH – OFF IGNITION – OFF MASTER SWITCH – ON Check fuel quantity (Order fuel, if needed) MASTER SWITCH – OFF FUEL - SELECTOR BOTH, GAGES CHECK doors/windwos Alternate Static Manifold Press - AMBIENT WINGS (BOTH) FLAPS FUEL TANK SUMP GEAR – tire inflation, tread, brakes and lines COOLING AIR INTAKES FUEL TANK – quantity, secure cap UPPER SURFACE OF WING WING STRUT LEADING EDGE PITOT TUBE STALL WARNING VENT (LEFT) FUEL TANK VENT WINGTIP, NAV/STROBE AILERON –security and freedom of movement LOWER SURFACE OF WING FUSELAGE REAR WINDOWS – cleanliness, damage BAGGAGE DOOR – SECURE ANTENNAS HORIZONTAL and VERTICAL STABILIZERS ELEVATOR and RUDDER NAV LIGHT (white)/BEACON/STROBE TAIL TIEDOWN TAILWHEEL– Cables, Cotter Pins, Tire Pressure NOSE WINDSHIELD COOLING AIR INTAKES OIL – CHECK quantity, 9 qts MINIMUM, secure cap FUEL STRAINER ENGINE COMPARTMENT PROPELLER, SPINNER ALTERNATOR BELT – CHECK proper tension LANDING LIGHTS CARB AIR FILTER STATIC PORT (Both sides of cowling) – CHECK COWL FLAPS |  | ADDITINOAL PRE-FLIGHT WEATHER WEIGHT & BALANCE NAVIGATION PLAN NAVIGATION CHARTS AND EQUIPMENT PERFORMANCE AIRCRAFT DATA Gross – 3175 lbs, Fuel 75 Gal Useable Oil 12 QTS Max, 9 QTX Min Tire Pressure: Tailwheel: 25-40 PSI, Main: 20-30 PSI DURING TAXI AVIONICS MASTER SWITCH – ON RADIOS – ON and set to desired frequencies TRANSPONDER – 1200 LIGHTS – As required BRAKES – CHECK THE MANTRA - Airspeed Indicator (ASI) – Indicates zero - Attitude Indicator (AI) – Is stable and deflects no more than 5° in a turn - Altimeter (ALT) – Within 75 feet on known altitude - Vertical Speed Indicator (VSI) – Indicates zero - Heading Indicator (HI) – Turns in the correct direction - Turn Coordinator (TC) – Turns in the correct direction, ball swings to the outside and has fluid - Compass – Swings freely, shows know headings and has fluid in it - Outside Air Temperature (OAT) – Reads correctly - Clock – Set to correct time! |  |
| EMERGENCY PROCEDURES – CESSNA 180K |  |  |  |
| ENGINE FAILURE DURING CRUISE - Best Glide - Find Landing Site - Carb Heat – ON - Primer – IN & LOCKED - Fuel Selector – BOTH - Mixture – RICH - Mags – ON OR START - DECLARE EMERGENCY – Current Freq. or 121.5 - Transponder 7700 BEST GLIDE (FLAPS UP) 2800 lbs – 75 KTS 2400 lbs – 70 KTS 2000 lbs – 65 KTS EMERGENCY LANDING – NO POWER - Airspeed – 75 KTS (flaps UP) 65 KTS (flaps DOWN) - DECLARE EMERGENCY - Seat Belts – SECURE - Mixture – IDLE CUTOFF - Fuel Selector – OFF - Mags – OFF - Transponder – 7700 - All Switches (Except Master) – OFF - Wing Flaps – AS REQUIRED - Master – OFF - Doors – UNLATCHED - Touchdown – TAIL LOW - FLY IT ‘TIL IT’S STOPPED! LOST COMMUNICATION – SQUAK 7600 VFR – Remain VFR, Land, Light Signals IFR Remain on route assigned, expected or direct to the fix if vectored in the last ATC clearance, or fly the route as filed in the flight plan. Remain at highest altitude assigned in clearance, or higher MEA. At clearance limit, begin descent & approach at filed or EFC time. |  | ENGINE FAILURE AFTER TAKEOFF - Best Glide - Mixture – Cut OFF - Fuel – OFF - Mags – OFF - Flaps – As Req’d - Falps DOWN @ 65 KTS - Doors – UNLATCHED - DECLARE EMERGENCY - Master – OFF ENGINE FIRE - Mixture – IDLE CUTOFF - Fuel –OFF - Cabin HEAT/AIR – OFF - Overhead Vents – OPEN - Lower Nose to put out fire - BEST GLIDE - DECLARE EMERGENCY – Current Freq. or 121.5 - Master OFF - Prepare for LANDING LOW VOLTAGE LGIHT ILLUMINATES DURING FLIGHT OR AMMETER INDICATES DISCHARGE - Avionics Power Switch – OFF - Alternator Circuit Breaker – CHECK IN - Master Switch – OFF (both sides) - Master Switch – ON - Low Voltage Light – CHECK OFF - Avionics Power Switch – ON If low voltage illuminates again… - Alternator – OFF - Non-req’d Radio and Elec – OFF - Flight – TERMINATE when able AMMETER SHOWS EXCESSIVE CHARGE - Master Switch Alternator Side – OFF - Alternator Circuit Breaker – PULL - Electrical Equipment – OFF - Flight – TERMINATE when able |  |
| GENERAL NOTES |  |  |  |
| IFR MANDATORY REPORTS: FAME Performance Fixes: Passing (in non-radar area): “Approach, Skywagon 2535K, Level 6000, SLI, Estimate PDZ in 16 min.” Altitude Changes: “Depart. 6000 for 4000” Missed Approach: “ Executing Published Missed, Diverting to XXX” Equipment: Loss or Problems Performance: Poor Climb, TAS Change 5% or 10 KTS IFR CLEARANCE (CRAFT) Clearance Routing Altitude Frequency Transponder Optimum Take-Off Prop Speeds Mach 0.88 0.9 0.92 Temp (C) RPM Low RPM Mid RPM High 30 2688 2749 2810 25 2664 2725 2786 20 2641 2701 2761 15 2618 2678 2737 10 2595 2654 2713 5 2572 2631 2689 0 2549 2607 2665 -5 2526 2583 2641 -10 2503 2560 2617 -15 2480 2536 2592 -20 2457 2513 2568 Assumes Prop is: 86 inches SECURING AIRCRAFT Record time Record squawks in maintenance log CONTROL LOCK COVERS TIE DOWNS / CHOCKS CLOSE FLIGHT PLAN!!! |  | REQUIRED EQUIPMENT – DAY VFR Tach; For each engine. Oil Temp; For each engine. Manifold Pressure; Const. Sp. Prop. Airspeed Indicator. Anti-Collision Lights. Oil Pressure; For each engine. Fuel Gage; For each tank. Altimeter. Magnetic Compass. ELT. Seatbelts. Stall Horn. ADDTL. REQ. EQUIPMENT – NIGHT VFR Fuses within Reach. Landing Light; If for hire. Beacon or Strobe. Position Lights. Source of Power. IFR Alternator / Source of Power. Radios. Attitude Indicator. Ball. Clock. Adjustable Sensitive Altimeter. Rate of Turn Indicator. Directional Gyro. DME over FL240. COMMON FREQUENCIES Emergency 121.5 7600 – Lost Comm FSS 122.0 7700 - Emergency |

| EMERGENCY PROCEDURES – CESSNA 180K |  |
| --- | --- |
| ENGINE FAILURE DURING CRUISE - Best Glide - Find Landing Site - Carb Heat – ON - Primer – IN & LOCKED - Fuel Selector – BOTH - Mixture – RICH - Mags – ON OR START - DECLARE EMERGENCY – Current Freq. or 121.5 - Transponder 7700 BEST GLIDE (FLAPS UP) 2800 lbs – 75 KTS 2400 lbs – 70 KTS 2000 lbs – 65 KTS EMERGENCY LANDING – NO POWER - Airspeed – 75 KTS (flaps UP) 65 KTS (flaps DOWN) - DECLARE EMERGENCY - Seat Belts – SECURE - Mixture – IDLE CUTOFF - Fuel Selector – OFF - Mags – OFF - Transponder – 7700 - All Switches (Except Master) – OFF - Wing Flaps – AS REQUIRED - Master – OFF - Doors – UNLATCHED - Touchdown – TAIL LOW - FLY IT ‘TIL IT’S STOPPED! LOST COMMUNICATION – SQUAK 7600 VFR – Remain VFR, Land, Light Signals IFR Remain on route assigned, expected or direct to the fix if vectored in the last ATC clearance, or fly the route as filed in the flight plan. Remain at highest altitude assigned in clearance, or higher MEA. At clearance limit, begin descent & approach at filed or EFC time. | ENGINE FAILURE AFTER TAKEOFF - Best Glide - Mixture – Cut OFF - Fuel – OFF - Mags – OFF - Flaps – As Req’d - Falps DOWN @ 65 KTS - Doors – UNLATCHED - DECLARE EMERGENCY - Master – OFF ENGINE FIRE - Mixture – IDLE CUTOFF - Fuel –OFF - Cabin HEAT/AIR – OFF - Overhead Vents – OPEN - Lower Nose to put out fire - BEST GLIDE - DECLARE EMERGENCY – Current Freq. or 121.5 - Master OFF - Prepare for LANDING LOW VOLTAGE LGIHT ILLUMINATES DURING FLIGHT OR AMMETER INDICATES DISCHARGE - Avionics Power Switch – OFF - Alternator Circuit Breaker – CHECK IN - Master Switch – OFF (both sides) - Master Switch – ON - Low Voltage Light – CHECK OFF - Avionics Power Switch – ON If low voltage illuminates again… - Alternator – OFF - Non-req’d Radio and Elec – OFF - Flight – TERMINATE when able AMMETER SHOWS EXCESSIVE CHARGE - Master Switch Alternator Side – OFF - Alternator Circuit Breaker – PULL - Electrical Equipment – OFF - Flight – TERMINATE when able |